# Profiler Wizard File Import Guide

# Overview

If the supplied profiling system requires a custom or updated code set, the following procedure should be used to import the supplied code.

## Importing Profiler Code:

#### Via the wizard (Recommended):

- Open the wizard and create/modify the appropriate file.

- Continue all the way through until the review page.

- Before clicking “Finish”, click the drop down to select the version and select “Import”

- Navigate to the folder containing the new files and click “Select Folder”

  - The folder should only contain a single .dlt and .dld file, any exceptions will create an error

  - Depending on PC settings, the folder may appear empty

- The wizard will search the files and retrieve the appropriate version and name it in the drop down menu

- As long as the imported version is higher, it will default to that in the future.

#### Manually adding files:

- Using windows file explorer, navigate to where the Profiler Wizard .exe is located

- There should be a folder in the same location labeled “campbellcode”

- Navigate to the folder labeled Profiler

- Create a new folder named with the appropriate version

  - The version can be found by opening the supplied .dlt file and finding the variable: progVersion

- Copy and paste the .dlt and .dld file into this folder

  - If improper/additional files are placed in the folder, the wizard may or may not be able to create the necessary/correct files

- Next time running the wizard, the new version will be available in the dropdown on the review page

## Importing MET Code:

#### Via the wizard (Recommended):

- Open the wizard and create/modify the appropriate file.

- Continue until you reach the page asking if you have MET sensors.

- If your system doesn’t have a MET sensor but does have some type of special communication output, click “Next” and perform the same steps on the next page of the wizard

- Select “Custom” and a few new sections should appear

- Under the “Custom Program Selection” section, select “Import” from the dropdown menu

- Navigate to the folder containing the new files and click “Select Folder”

  - The folder should only contain a single .dlt and .dld file, any exceptions will create an error

  - Depending on PC settings, the folder may appear empty

- The wizard will search the files and retrieve the appropriate version and name it in the drop down menu

#### Manually adding files:

- Using windows file explorer, navigate to where the Profiler Wizard .exe is located

- There should be a folder in the same location labeled “campbellcode”

- Navigate to the folder labeled MET

- Navigate to the appropriate MET suite folder, if no meteorological sensors are present, it is recommended to place it in the SpComms folder. If unsure use Custom folder

- Create a new folder named with the appropriate version

  - The version can be found by opening the supplied .dlt file and finding the variable: progVersion

- Copy and paste the .dlt and .dld file into this folder

  - If improper/additional files are placed in the folder, the wizard may or may not be able to create the necessary/correct files

- Next time running the wizard, it will be present in the custom dropdown  if placed there, otherwise, selecting the corresponding type will use the correct files

## Removing Files:

#### If there are incorrect or undesired versions present, they can be removed as options.

Follow the same steps as manually adding the files, except instead of pasting the folder, delete the folder with the code and next time the wizard is ran, those files will not be available

*** The various options of met suites/ special communication methods will still be available as options if their folders are deleted, but wizard will show an error message when trying to create files if they are missing

## Additional Notes:

#### Retain default settings:

If there are settings that the wizard overwrites or are a variable in the settings file that are not present in the wizard, and a different from default value is desired, the .dlt file can be updated accordingly and the wizard will not change the values.

- Open the .dlt file

- Find the variable that needs modified

- Modify the value to new desired value

  - Stopping at this step will change default values but can still be overwritten by the wizard

- After the value, there will be a string in the format of: ‘#______#

- Placing a space between the ‘#, making it ‘ #___#, will cause the wizard to skip over that variable when creating the settings file

  - If a value is not changing that should be, verify the formatting of ‘#_____# is followed